Guangcheng Technology

USB CAN Interface Function Library

（ECanVci.dll）

Instruction Manual   V5.2

| Revision History |  |  |
| --- | --- | --- |
| Version | Date | Reason |
| V5.1 | 2013/6/16 | Create a document |
| V5.2 | 2015/9/10 | Add some parameters |

Contents

1 Explanation of the function and use................................................................................................1

1.1 Device type definition.......................................................................................................... 1

1.2 Error codes definition ..........................................................................................................1

1.3 Function structure..................................................................................................................2

1.3.1 BOARD_INFO.......................................................................................................... 2

1.3.2 CAN_OBJ.................................................................................................................. 3

1.3.3 CAN_STATUS........................................................................................................... 4

1.3.4 ERR_INFO.................................................................................................................5

1.3.5 INIT_CONFIG ......................................................................................................... 5

1.3.6 FILTER_RECORD .................................................................................................. 8

1.4 Function description.............................................................................................................. 8

1.4.1 OpenDevice................................................................................................................ 9

1.4.2 CloseDevice ........................................................................................................... 10

1.4.3 InitCan .................................................................................................................... 11

1.4.4 ReadBoardInfo ....................................................................................................... 13

1.4.5 ReadErrInfo.............................................................................................................. 14

1.4.6 ReadCanStatus   ..................................................................................................... 19

1.4.7 GetReference ..........................................................................................................20

1.4.8 SetReference............................................................................................................ 21

1.4.9 GetReceiveNum....................................................................................................... 22

1.4.10 ClearBuffer ...........................................................................................................23

1.4.11 StartCAN .............................................................................................................. 24

1.4.12 Transmit..................................................................................................................25

1.4.13 Receive ................................................................................................................. 26

1.4.14 ResetCAN ............................................................................................................ 27

1.5 Method of use function....................................................................................................... 28

1.5.1 VC call the methods using dll.................................................................................. 28

1.5.2 VB call the methods using dll.................................................................................. 28

1.6 Interface function library.................................................................................................... 29

Sales and service............................................................................................................................... 30

1 Explanation of the function and use

1.1 Device type definition

| Device name | Device type number |
| --- | --- |
| USBCAN-I | 3 |
| USBCAN-II | 4 |

| Name | Value | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ERR_CAN_OVERFLOW | 0x00000001 | CAN controller |  |  |  |
|  |  | FIFO |  | overflow |  |
| ERR_CAN_ERRALARM | 0x00000002 | CAN controller error alarm |  |  |  |
| ERR_CAN_PASSIVE | 0x00000004 | CAN controller passive error |  |  |  |
| ERR_CAN_LOSE | 0x00000008 | CAN controller arbitration lose |  |  |  |
| ERR_CAN_BUSERR | 0x00000010 | CAN controller bus error |  |  |  |
| ERR_CAN_REG_FULL | 0x00000020 |  | CAN receive register full |  |  |
| ERR_CAN_REC_OVER | 0x00000040 | CAN receive register overflow |  |  |  |
| ERR_CAN_ACTIVE | 0x00000080 | CAN controller active error |  |  |  |
| ERR_DEVICEOPENED | 0x00000100 | Device already open |  |  |  |
| ERR_DEVICEOPEN | 0x00000200 | Open device error |  |  |  |
| ERR_DEVICENOTOPEN | 0x00000400 | Device did not open |  |  |  |
| ERR_BUFFEROVERFLOW | 0x00000800 | Buffer overflow |  |  |  |
| ERR_DEVICENOTEXIST | 0x00001000 | Device not exist |  |  |  |
| ERR_LOADKERNELDLL | 0x00002000 | Load dll failure |  |  |  |
| ERR_CMDFAILED | 0x00004000 | Execute the command failure error code |  |  |  |
| ERR_BUFFERCREATE | 0x00008000 | insufficient memory |  |  |  |

1.2 Error codes definition

1.3 Function structure

1.3.1 BOARD_INFO Description

BOARD_INFO structure containing ECAN Series Device.Structure will be fulled in function ReadBoardInfo.

typedef struct _BOARD_INFO { USHORT hw_Version; USHORT fw_Version; USHORT dr_Version; USHORT in_Version; USHORT irq_Num;

BYTE      can_Num;

CHAR     str_Serial_Num[20]; CHAR     str_hw_Type[40]; USHORT Reserved[4];

} BOARD_INFO, *P_BOARD_INFO;

Members:

hw_Version

Hardware version number，hexadecimal representation.

fw_Version

Firmware version number，hexadecimal representation.

dr_Version

Driver version number，hexadecimal representation.

in_Version

Interface library version number，hexadecimal representation.

irq_Num

System reserved。

can_Num

Represents the total number of CAN channel.

str_Serial_Num

This board card's serial number.

str_hw_Type

Hardware type.

Reserved

System reserved.

1.3.2 CAN_OBJ

Description

In  the functions transmit and receive,CAN_OBJ  structure is used to transmit

CAN message frame.

typedef struct _CAN_OBJ { UINT ID;

UINT TimeStamp; BYTE TimeFlag; BYTE SendType; BYTE RemoteFlag; BYTE ExternFlag; BYTE DataLen; BYTE Data[8]; BYTE Reserved[3];

} CAN_OBJ, *P_CAN_OBJ;

Members:

ID

Message identifier.

TimeStamp

Receiving the stamp information of the frame ，start timing when the CAN

controller is initialized，the unit is 0.1ms.

TimeFlag

In terms of whether to use the time stamp ，1 is the effective TimeFlag and

TimeStamp are only meaningful when the frame is received.

SendType

Sending type. =0 indicates Normal type，=1 indicates Single Send

RemoteFlag

Whether it is a remote flag.=1 indicates remote flag，=0 indicates data flag.

ExternFlag

Whether it is a extern flag.=1 indicates extern flag,=0 indicates standard flag.

DataLen

Date length (<=8)，that is,the length of date.

Data

Packet data.

Reserved

System reserved.

1.3.3 CAN_STATUS

Description

CAN_STATUS structure includes CAN controller status information.Structure will be fulled in function ReadCanStatus.

typedef struct _CAN_STATUS { UCHAR ErrInterrupt; UCHAR regMode;

UCHAR regStatus; UCHAR regALCapture; UCHAR regECCapture; UCHAR regEWLimit; UCHAR regRECounter; UCHAR regTECounter; DWORD Reserved;

} CAN_STATUS, *P_CAN_STATUS;

Members:

ErrInterrupt

Interrupt record,Reading operation will be cleared.

regMode

CAN controller mode register.

regStatus

CAN controller state register.

regALCapture

CAN controller arbitration lose register.

regECCapture

CAN controller error register.

regEWLimit

CAN controller error alarm limit register.

regRECounter

CAN controller receive error register.

regTECounter

CAN controller transmit error register.

Reserved

System reserved.

1.3.4 ERR_INFO Description

ERR_INFO structure is used to load VCI library error information.Structure will be fulled in function ReadErrInfo.

typedef struct _ERR_INFO { UINT ErrCode;

BYTE Passive_ErrData[3]; BYTE ArLost_ErrData;

} ERR_INFO, *P_ERR_INFO;

Members:

ErrCode

Error code,corresponding 1.2 the definition of error code

Passive_ErrData

When  mistakes  have  negative  error  is  expressed  as  a  negative  error  error identification data.

ArLost_ErrData

When the error of arbitration missing error is expressed as lost the wrong mistake identification data.

1.3.5 INIT_CONFIG

Description

INIT_CONFIG structure defines the initialization configuration of the CAN.The structure will be filled in InitCan function.

typedef struct _INIT_CONFIG { DWORD AccCode; DWORD AccMask; DWORD Reserved;

UCHAR Filter; UCHAR Timing0; UCHAR Timing1; UCHAR Mode;

} INIT_CONFIG, *P_INIT_CONFIG;

Members:

AccCode

Receive filtered acceptance code.

AccMask

Receive filter mask.

Reserved

Reserved.

Filter

Filtering method,allowing setting 0 and 1,0 is close the filter,1 is open the filter.

Timing0

Baud rate timer 0(BTR0).

Timing1

Baud rate timer 1(BTR1).

Mode

Operating mode,  0=normal  operation,  1=listen-only  mode, 2=spontaneous admission and sending test mode.

Remarks:

Timing0 and Timing1 are used to set baud rate.Here are several kinds of common baud rate:

| CAN baud rate | Btr0 | Btr1 |
| --- | --- | --- |
| 5Kbps | 0xBF | 0xFF |
| 10Kbps | 0x31 | 0x1C |
| 20Kbps | 0x18 | 0x1C |
| 40Kbps | 0x87 | 0xFF |
| 50Kbps | 0x09 | 0x1C |
| 80Kbps | 0x83 | 0XFF |
| 100Kbps | 0x04 | 0x1C |
| 125Kbps | 0x03 | 0x1C |
| 200Kbps | 0x81 | 0xFA |
| 250Kbps | 0x01 | 0x1C |
| 400Kbps | 0x80 | 0xFA |
| 500Kbps | 0x00 | 0x1C |
| 666Kbps | 0x80 | 0xB6 |
| 800Kbps | 0x00 | 0x16 |
| 1000Kbps | 0x00 | 0x14 |

1.3.6 FILTER_RECORD

Description

FILTER_RECORD structure defines the CAN filter range.The structure will be filled in SetReference function.

typedef struct _FILTER_RECORD{ DWORD ExtFrame;

DWORD Start; DWORD End;

} FILTER_RECORD,*P_FILTER_RECORD;

Members:

ExtFrame

Filter frame type logo.1=extended frame,0=standard Frame.

Start

The filter range initial frame ID.

End

The end of the filter range frame ID.

1.4 Function description

1.4.1 OpenDevice

Description

The function is used to connect devices.

DWORD     stdcall OpenDevice(DWORD DevType,DWORD  DevIndex, DWORD Reserved);

Parameters：

DevType

Device type. 3=USBCAN I. 4=USBCAN II.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

Reserved

Retention parameters.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; /* USBCAN-I */

int nDeviceInd = 0; int nReserved =0; DWORD dwRel;

dwRel = OpenDevice(nDeviceType, nDeviceInd, nReserved);

if (dwRel != STATUS_OK)

{

MessageBox(_T("fail          to          open          the          device!"),_T("warning"), MB_OK|MB_ICONQUESTION);

return FALSE;

}

1.4.2 CloseDevice

Description

| unction | is | used | to | close |
| --- | --- | --- | --- | --- |

The f

the connection.

DWORD    stdcall CloseDevice(DWORD DevType, DWORD DevIndex);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; // USBCAN-I

int nDeviceInd = 0; // Device index 0

BOOL bRel;

bRel = CloseDevice(nDeviceType, nDeviceInd);

1.4.3 InitCan

Description

| unction | is | used | to | initialize |
| --- | --- | --- | --- | --- |

The f

the specified CAN.

DWORD      stdcall  InitCan(DWORD   DevType,   DWORD  DevIndex,  DWORD CANIndex, P_INIT_CONFIG pInitConfig);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index,0=CAN,1=CAN1.

pInitConfig

Initialization parameter structure.

Member                      Functional description

pInitConfig->AccCode        AccCode corresponds to SJA1000 four registers ACR0 ，

ACR1，ACR2，ACR3，high type corresponding to ACR0，

pInitConfig->AccMask

low type corresponding to ACR3；AccMask corresponding to SJA1000 four registers AMR0，AMR1，AMR2，AMR3， high type corresponding to AMR0，low type corresponding to AMR3。（After look at the table）

pInitConfig->Reserved        Reserved.

pInitConfig->Filter              Filtering method，0 is close the filter,1 is open the filter. pInitConfig->Timing0         Baud rate timer 0

pInitConfig->Timing1         Baud rate timer 1

pInitConfig->Mode              Operating mode,0=normal mode，1=listen-only mode.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; //    USBCAN-I int nDeviceInd = 0; //    Device index 0 int nCANInd = 0;

int nReserved = 0; DWORD dwRel;

dwRel = OpenDevice(nDeviceType, nDeviceInd, nReserved);

if (dwRel != STATUS_OK)

{

MessageBox(_T("fail to open the device!"), _T("warning"),

MB_OK|MB_ICONQUESTION);

}

1.4.4 ReadBoardInfo

Description

| unction | is | used | to | read |
| --- | --- | --- | --- | --- |

The f

the adapter hardware information.

DWORD       stdcall    ReadBoardInfo(DWORD    DevType,    DWORD    DevIndex, P_BOARD_INFO pInfo);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

pInfo

BOARD_INFO is used to store device information structure pointer.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; INIT_CONFIG vic; BOARD_INFO vbi; DWORD dwRel;

dwRel = ReadBoardInfo(nDeviceType, nDeviceInd, nCANInd, &vbi);

1.4.5 ReadErrInfo

Description

The function is used to attain the last error information of the adapter.

DWORD     stdcall ReadErrInfo(DWORD DevType, DWORD DevIndex, DWORD CANIndex, P_ERR_INFO pErrInfo);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index

pErrInfo

ERR_INFO is used to store error information structure pointer. pErrInfo->ErrCode may be one of the following error code combination：

| ErrCode | Passive_ErrData | ArLost_ErrData | Error description |  |
| --- | --- | --- | --- | --- |
| 0x0100 | N/A | N/A | Device already open |  |
| 0x0200 | N/A | N/A | Open device error |  |
| 0x0400 | N/A | N/A | Device did not open |  |
| 0x0800 | N/A | N/A | Buffer overflow |  |
| 0x1000 | N/A | N/A | Device not exist |  |
| 0x2000 | N/A | N/A | Load dll failure |  |
| 0x4000 | N/A | N/A | Execute the command failure error |  |
| 0x8000 | N/A | N/A | insufficient memory |  |
| 0x0001 | N/A | N/A | CAN controller |  |
|  |  |  | FIFO | overflow |
| 0x0002 | N/A | N/A | CAN controller error alarm |  |
| 0x0004 | Refer to next table | N/A | CAN controller passive error |  |
| 0x0008 | N/A | Refer to next table | CAN controller arbitration lose |  |
| 0x0010 | N/A | N/A | CAN controller bus error |  |
| 0x0020 |  |  | CAN receive register full |  |
| 0x0040 |  |  | CAN receive register overflow |  |
| 0x0080 |  | Refer to next table | CAN controller active error |  |

Returns:

1=success,0=failure.

Notes

If (PErrInfo->ErrCode&0x0004)==0x0004,there is CAN controller passive error.

| rror | code | capture | function |
| --- | --- | --- | --- |

PErrInfo->Passive_ErrData[0] E

Bit7       Bit6           Bit5          Bit4       Bit3         Bit2         Bit1         Bit0

Error code type

Error

attribute                               Error section

| The | error | code | type | function |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Bit ECC.7 |  | Bit ECC.6 | Function |
|  |  |  |  | 0 |  | 0 | Bit error |
|  |  |  |  | 0 |  | 1 | Format error |
|  |  |  |  | 1 |  | 0 | Filling error |
|  |  |  |  | 1 |  | 1 | Other errors |

Error attribute

bit5 =0； transmit error bit5 =1； receive error

| Error | section |  | function |  | description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit4 |  | Bit 3 |  | Bit 2 |  | Bit 1 | Bit 0 | Function |
|  |  | 0 |  | 0 |  | 0 |  | 1 | 1 | Start of frame |
|  |  | 0 |  | 0 |  | 0 |  | 1 | 0 | ID.28-ID.21 |
|  |  | 0 |  | 0 |  | 1 |  | 1 | 0 | ID.20-ID.18 |
|  |  | 0 |  | 0 |  | 1 |  | 0 | 0 | SRTR bit |
|  |  | 0 |  | 0 |  | 1 |  | 0 | 1 | IDE bit |
|  |  | 0 |  | 0 |  | 1 |  | 1 | 1 | ID.17-ID.13 |
|  |  | 0 |  | 1 |  | 1 |  | 1 | 1 | ID.12-ID.5 |
|  |  | 0 |  | 1 |  | 1 |  | 1 | 0 | ID.4-ID.0 |
|  |  | 0 |  | 1 |  | 1 |  | 0 | 0 | RTR bit |
|  |  | 0 |  | 1 |  | 1 |  | 0 | 1 | Reserved bit 1 |
|  |  | 0 |  | 1 |  | 0 |  | 0 | 1 | Reserved bit 0 |
|  |  | 0 |  | 1 |  | 0 |  | 1 | 1 | Data length code |
|  |  | 0 |  | 1 |  | 0 |  | 1 | 0 | Data area |
|  |  | 0 |  | 1 |  | 0 |  | 0 | 0 | CRC sequence |
|  |  | 1 |  | 1 |  | 0 |  | 0 | 0 | CRC delimiter |
|  |  | 1 |  | 1 |  | 0 |  | 0 | 1 | Reply channel |
|  |  | 1 |  | 1 |  | 0 |  | 1 | 1 | Reply delimiter |
|  |  | 1 |  | 1 |  | 0 |  | 1 | 0 | End of frame |
|  |  | 1 |  | 0 |  | 0 |  | 1 | 0 | Discontinue |
|  |  | 1 |  | 0 |  | 0 |  | 0 | 1 | Activity error sign |
|  |  | 1 |  | 0 |  | 1 |  | 1 | 0 | Negative error sign |
|  |  | 1 |  | 0 |  | 0 |  | 1 | 1 | Control biterror |
|  |  | 1 |  | 0 |  | 1 |  | 1 | 1 | Error delimiter |
|  |  | 1 |  | 1 |  | 1 |  | 0 | 0 | Overflow mark |

| Receive | error |  |  | counter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmit |  | error |  |  | counter |  |  |  |  |
| 8)==0x0008, |  |  | there |  |  | exist |  | CAN | controller |

PErrInfo->Passive_ErrData[1] PErrInfo->Passive_ErrData[2] IF(PErrInfo->ErrCode&0x000 arbitration lost error.

PErrInfo->ArLost_ErrData    Arbitration missing code target-catch bit

| function. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit7 |  | Bit6 | Bit5 | Bit4 | Bit3 |  | Bit2 |  | Bit1 |  | Bit0 |
|  | ——— |  | ——— | ——— |  |  | Error |  | section |  |  |  |

|  |  | Bit |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ALC. 4 | ALC. 3 | ALC. 2 | ALC. 1 | ALC. 0 | value | Function |
| 0 | 0 | 0 | 0 | 0 | 0 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit1 |
| 0 | 0 | 0 | 0 | 1 | 1 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit2 |
| 0 | 0 | 0 | 1 | 0 | 2 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit3 |
| 0 | 0 | 0 | 1 | 1 | 3 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit4 |
| 0 | 0 | 1 | 0 | 0 | 4 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit5 |
| 0 | 0 | 1 | 0 | 1 | 5 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit6 |
| 0 | 0 | 1 | 1 | 0 | 6 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit7 |
| 0 | 0 | 1 | 1 | 1 | 7 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit8 |
| 0 | 1 | 0 | 0 | 0 | 8 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit9 |
| 0 | 1 | 0 | 0 | 1 | 9 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit10 |
| 0 | 1 | 0 | 1 | 0 | 10 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit11 |
| 0 | 1 | 0 | 1 | 1 | 11 | Arbitration lost in SRTR |
| 0 | 1 | 1 | 0 | 0 | 12 | Arbitration lost in IDE |
| 0 | 1 | 1 | 0 | 1 | 13 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit12 |
| 0 | 1 | 1 | 1 | 0 | 14 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit13 |
| 0 | 1 | 1 | 1 | 1 | 15 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit14 |

Decimal

| 1 | 0 | 0 | 0 | 0 | 16 | Arbitration lost in |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | identification code bit15 |
| 1 | 0 | 0 | 0 | 1 | 17 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit16 |
| 1 | 0 | 0 | 1 | 0 | 18 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit17 |
| 1 | 0 | 0 | 1 | 1 | 19 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit18 |
| 1 | 0 | 1 | 0 | 0 | 20 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit19 |
| 1 | 0 | 1 | 0 | 1 | 21 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit20 |
| 1 | 0 | 1 | 1 | 0 | 22 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit21 |
| 1 | 0 | 1 | 1 | 1 | 23 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit22 |
| 1 | 1 | 0 | 0 | 0 | 24 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit23 |
| 1 | 1 | 0 | 0 | 1 | 25 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit24 |
| 1 | 1 | 0 | 1 | 0 | 26 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit25 |
| 1 | 1 | 0 | 1 | 1 | 27 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit26 |
| 1 | 1 | 1 | 0 | 0 | 28 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit27 |
| 1 | 1 | 1 | 0 | 1 | 29 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit28 |
| 1 | 1 | 1 | 1 | 0 | 30 | Arbitration lost in |
|  |  |  |  |  |  | identification code bit29 |
| 1 | 1 | 1 | 1 | 1 | 31 | Arbitration lost in ERTR |

E.g:

#include "ECanVci.h"

int nDeviceType =3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; ERR_INFO vei; DWORD dwRel;

dwRel = ReadErrInfo(nDeviceType, nDeviceInd, nCANInd, &vei);

1.4.6 ReadCanStatus

Description

The function is used to attain CAN status.

DWORD       stdcall    ReadCanStatus(DWORD    DevType,    DWORD    DevIndex, DWORD CANIndex, P_CAN_STATUS pCANStatus);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

pCANStatus

Using to store CAN status CAN_STATUS structure pointer.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 4; // USBCAN-II

int nDeviceInd = 0; int nCANInd = 0; INIT_CONFIG vic; CAN_STATUS vcs; DWORD dwRel;

dwRel = ReadCanStatus(nDeviceType, nDeviceInd, nCANInd, &vcs);

1.4.7 GetReference

Description

The function is used to attain device parameter.

DWORD     stdcall GetReference(DWORD DevType, DWORD DevIndex, DWORD CANIndex, DWORD RefType, PVOID pData);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

RefType

Reference type.

pData

| ata | buffer | address | first | pointer. |
| --- | --- | --- | --- | --- |

Using to store reference in d

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 4; //    USBCAN-II

int nDeviceInd = 0; int nCANInd = 0; BYTE info[14]; DWORD dwRel; info[0] = 1;

dwRel = GetReference(nDeviceType, nDeviceInd, nCANInd, 1, (PVOID)info);

1.4.8 SetReference

Description

| unction | is | used | to |
| --- | --- | --- | --- |

The f

set the corresponding parameters of the device.

DWORD     stdcall SetReference(DWORD DevType, DWORD DevIndex, DWORD CANIndex, DWORD RefType, PVOID pData);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

RefType

Reference type.

pData

| buffer | address | first | pointer. |
| --- | --- | --- | --- |

Using to store reference in data

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; BYTE baud; DWORD dwRel; baud = 0;

dwRel = SetReference(nDeviceType, nDeviceInd, nCANInd, 1, (PVOID)baud);

1.4.9 GetReceiveNum

Description

| The f | unction |  | is | used |  | to |
| --- | --- | --- | --- | --- | --- | --- |
| designated |  | receiving |  |  | buffer. |  |

specify the received but has not been read frames in the

ULONG       stdcall   GetReceiveNum(DWORD   DevType,   DWORD   DevIndex, DWORD CANIndex);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

Returns:

Return frames that have not been read yet.

E.g:

#include "ECanVci.h"

int nDeviceType =3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; DWORD dwRel;

dwRel = GetReceiveNum(nDeviceType, nDeviceInd, nCANInd);

1.4.10 ClearBuffer

Description

The function is used to clear the receive and send buffer of the designated channel specified by device.

DWORD     stdcall ClearBuffer(DWORD DevType, DWORD DevIndex, DWORD CANIndex);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 3; // USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; DWORD dwRel;

dwRel = ClearBuffer(nDeviceType, nDeviceInd, nCANInd);

1.4.11 StartCAN

Description

| The f |  | unction is used |  | to open the device of |  |  |  |  | a particular can channel.If | there are |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| multiple | channels,you |  | should |  | use | many | times. |  |  |  |

DWORD      stdcall  StartCAN(DWORD  DevType,  DWORD  DevIndex,  DWORD CANIndex);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType =3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; int nReserved = 0; INIT_CONFIG vic; DWORD dwRel;

dwRel = OpenDevice(nDeviceType, nDeviceInd, nReserved);

if (dwRel != STATUS_OK)

{

MessageBox(_T("fail to open the device!"), _T("warning"), MB_OK|MB_ICONQUESTION);

return FALSE;

}

dwRel = InitCAN(nDeviceType, nDeviceInd, nCANInd, &vic);

if (dwRel == STATUS_ERR)

{

CloseDevice(nDeviceType, nDeviceInd); MessageBox(_T("fail to open the device!"), _T("warning"), MB_OK|MB_ICONQUESTION);

return FALSE;

}

dwRel = StartCAN(nDeviceType, nDeviceInd, nCANInd);

if (dwRel == STATUS_ERR)

{

CloseDevice(nDeviceType, nDeviceInd);

MessageBox(_T("fail to open the device!"), _T("warning"), MB_OK|MB_ICONQUESTION);

return FALSE;

}

1.4.12 Transmit

Description

The function is used to send CAN message frame.

ULONG      stdcall  Transmit(DWORD  DevType,  DWORD  DevIndex,  DWORD CANIndex, P_CAN_OBJ pSend, ULONG Len);

Parameters:

DevType

Device type.

DevIndex

Device  index,for  example,when  there  is  only  one  USBCAN,the  index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

pSend

The first address of the data frame arrays that have to be sent.

Len

The number of the data frames that have to be sent.

Returns:

Return the actual number of frames already sent.

E.g:

#include "ECanVci.h"

#include <string.h>

int nDeviceType = 3; //    USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; DWORD dwRel; CAN_OBJ vco;

ZeroMemory(&vco, sizeof (CAN_OBJ));

vco.ID = 0x00000000; vco.SendType = 0; vco.RemoteFlag = 0; vco.ExternFlag = 0; vco.DataLen = 8;

dwRel = Transmit(nDeviceType, nDeviceInd, nCANInd, &vco, i);

1.4.13 Receive

Description

The function is used to request reception.

ULONG      stdcall   Receive(DWORD   DevType,   DWORD   DevIndex,  DWORD CANIndex, P_CAN_OBJ pReceive, ULONG Len, INT WaitTime=－1);

Parameters:

DevType

Device type.

DevIndex

Device index,for example,when there is only one USBCAN,the index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

pReceive

To receive the first sent of the data frames.

Len

The length of the array used to receive data frames.

WaitTime

Wait timeout,in milliseconds.

Returns:

Return the number of frames that actually have been read.

E.g:

#include "ECanVci.h"

#include <string.h>

int nDeviceType = 3; // USBCAN-I

int nDeviceInd = 0; int nCANInd = 0; DWORD dwRel; CAN_OBJ vco[100];

dwRel = Receive(nDeviceType, nDeviceInd, nCANInd, vco, 100, 10);

1.4.14 ResetCAN

Description

The function is used to reset can.If USBCAN is bus off,you can call the function.

DWORD     stdcall  ResetCAN(DWORD  DevType,  DWORD  DevIndex,  DWORD CANIndex);

Parameters:

DevType

Device type.

DevIndex

Device index,for example,when there is only one USBCAN,the index number is 0, when there is two USBCAN,the index number is 0 and 1.

CANIndex

CAN channel index.

Returns:

1=success,0=failure.

E.g:

#include "ECanVci.h"

int nDeviceType = 4;//    USBCAN-II

int nDeviceInd = 0; int nCANInd = 0; DWORD dwRel;

dwRel = ResetCAN(nDeviceType, nDeviceInd, nCANInd);

1.5 Method of use function

First,put the library files in the working directory.There are three files in the library file：ECanVci.h、ECanVci.lib、ECanVci.dll.

1.5.1 VC call the method of dynamic libraries

| contains | ECANVCI. | h | header |
| --- | --- | --- | --- |

(1) In extension. CPP file

| connector | Settings | to | connect | to |
| --- | --- | --- | --- | --- |

E.g：#include “ECANVci.h” (2) In the engineering of the

file.

the ECANVCI. lib file.

1.5.2 VB call the method of dynamic libraries

Through the following method after the statement can call.

grammar:

[Public  |  Private]  Declare  Function   name  Lib   "libname"   [Alias  "aliasname"] [([arglist])] [As type]

Declare statement of the grammar contains the following sections:

Public (optional)

Used to declare all process can be used in all modules of function.

Private (optional)

Used to declare only contains the module used in the function of the statement.

Name（required）

Any legal function name.Dynamic link library entry points case sensitive.

Libname（required）

Containing the statement as a function of dynamic link library name or code resource name.

Alias（optional）

Said to be the function called the name of the other in the dynamic link library.When the external function and a function name repetition,you can use this parameter.When  the dynamic  link  library  functions  with  the  same  within  the scope of the public the names of the variables, constants, or any other process at the same time you can also use Alias.If a character of the dynamic link library function   is   not   in   conformity   with   the   dynamic   link   library   naming convention,you can also use Alias.

Aliasname（optional）

The dynamic link library.If the first character is not digital symbols (#),aliasname will be aliasname will be the name of the function at the entrance, in the dynamic link library.If the first character is (#) ，the serial number at the entrance of the subsequent characters must specify the function.

Arglist（optional）

On behalf of this function is called when the need to pass a parameter variables.

Type（optional）

The function returns the value of the data type；could be Byte、Boolean、Integer、

Long、Currency、Single、Double、Decimal（It is not supported）、Date、String

（Only support longer）or Variant，User-defined types, or object type.

arglist parameters of the grammar：

[Optional] [ByVal | ByRef] [ParamArray] varname[( )] [As type]

Part of the description：

Optional

。

，

parameter is not required  If you use this option    the subsequent parameters in arglist are required must be optional，Statement and must use Optional keywords.If you use the ParamArray，you could not use any Optional parameters.

ByVal（optional）

Said that the parameters are passed by value.

| （ | optional | ） |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| parameters |  |  | according |  | to | the | address. |

ByRef

Said the

E.g：

Public Declare Function OpenDevice Lib "ECANVCI" (ByVal devicetype As Long, ByVal deviceind As Long, ByVal reserved As Long) As Long

1.6 Interface library functions

OpenDevice(DWORD DeviceType,DWORD DeviceInd,DWORD Reserved); CloseDevice(DWORD DeviceType,DWORD DeviceInd);

InitCAN(DWORD DeviceType, DWORD DeviceInd, DWORD CANInd, P_INIT_CONFIG pInitConfig); ReadBoardInfo(DWORD DeviceType,DWORD DeviceInd,P_BOARD_INFO pInfo); ReadErrInfo(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,P_ERR_INFO pErrInfo);

ReadCANStatus(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,P_CAN_STATUS pCANStatus); GetReference(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,DWORD RefType,PVOID pData); SetReference(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,DWORD RefType,PVOID pData); GetReceiveNum(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd);

ClearBuffer(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd); StartCAN(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd); ResetCAN(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd);

Transmit(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,P_CAN_OBJ pSend,ULONG Len); Receive(DWORD DeviceType,DWORD DeviceInd,DWORD CANInd,P_CAN_OBJ pReceive,ULONG Len,INT WaitTime);

Sales and Service

Shenyang Guangcheng Technology CO, LTD

Address:  ChongShan  Middle  Road  No.  42,  Huanggu

District, Shenyang, Liaoning Province

Zip code：110000

Tel：024-31230060

Fax：024-31230070

URL：www.gcgd.net